# Project Documentation: ESP32 LCD Image Transfer (Serial Protocol)

## 1. Overview and Methodology

The final working solution utilizes a robust Serial Communication Protocol to transfer the raw image data over the USB cable (COM port). This bypasses the unreliable Wi-Fi/network stack for stability.

Protocol Details:

- Baud Rate: Set to a stable 115200.

- Trigger: The Python script first sends the command string "START_IMAGE_TRANSFER\n".

- Drawing: The ESP32's loop() function waits for this command, and upon receiving it, enters a blocking state to read exactly 108,800 bytes (the raw image data) directly from the Serial buffer and draws it line-by-line to the screen.

## 2. Image File Specification

The image file (myimage.raw) must be in the correct format for the display:

- Format: Raw binary data.

- Color Depth: 16-bit RGB565 (2 bytes per pixel).

- Dimensions: 320 pixels (width) x 170 pixels (height).

- Total Size: $320 \times 170 \times 2 = 108,800$ bytes.

- Byte Order: The data is assumed to be in the correct order for the TFT_eSPI library, which is why the manual byte-swap was removed.

## 3. Final ESP32 Sketch (Arduino IDE)

Replace your entire sketch content with this code.

C++

#include <TFT_eSPI.h>

TFT_eSPI tft = TFT_eSPI();

// --- IMAGE & GEOMETRY SPECIFICATIONS ---

#define IMAGE_WIDTH  320

#define IMAGE_HEIGHT 170

#define TFT_BL       32

#define TFT_BACKLIGHT_ON HIGH

// Total size in bytes (320 * 170 * 2 bytes/pixel)

const size_t EXPECTED_IMAGE_SIZE = 320 * 170 * 2; // 108800 bytes

const size_t LINE_BYTE_COUNT = IMAGE_WIDTH * 2; // 640 bytes per line

// Buffer for one line of image data (640 bytes)

uint8_t lineBuf[LINE_BYTE_COUNT];

#define STABLE_BAUD_RATE 115200

const char* START_COMMAND = "START_IMAGE_TRANSFER";

void drawImageFromSerial() {

Serial.println("STARTING DRAW: Data stream detected.");

tft.fillScreen(TFT_BLACK);

tft.setAddrWindow(0, 0, IMAGE_WIDTH, IMAGE_HEIGHT);

size_t bytesReadTotal = 0;

// Read and draw line by line (170 rows exactly)

for (int y = 0; y < IMAGE_HEIGHT; y++) {

// Blocking read: waits up to 1000ms for a full line (640 bytes) of data

size_t bytesRead = Serial.readBytes((char*)lineBuf, LINE_BYTE_COUNT);

if (bytesRead != LINE_BYTE_COUNT) {

Serial.printf("FATAL ERROR: Incomplete read at row %d. Expected %u bytes, got %u. Aborting.\n", y, LINE_BYTE_COUNT, bytesRead);

tft.fillScreen(TFT_RED);

return;

}

// Push the colors directly. The 'true' flag handles any necessary byte swapping.

tft.pushColors((uint16_t*)lineBuf, IMAGE_WIDTH, true);

bytesReadTotal += bytesRead;

}

if (bytesReadTotal == EXPECTED_IMAGE_SIZE) {

Serial.println("Image drawn successfully!");

} else {

Serial.printf("FATAL ERROR: Final size mismatch. Expected %u, Read %u.\n", EXPECTED_IMAGE_SIZE, bytesReadTotal);

tft.fillScreen(TFT_MAGENTA);

}

}

void setup() {

Serial.begin(STABLE_BAUD_RATE);

Serial.setTimeout(1000);

// --- DISPLAY INITIALIZATION ---

tft.init();

tft.setRotation(3);

tft.invertDisplay(true);

tft.fillScreen(TFT_BLACK);

pinMode(TFT_BL, OUTPUT);

digitalWrite(TFT_BL, TFT_BACKLIGHT_ON);

Serial.println("System Initialized. Awaiting Command...");

tft.drawString("Awaiting Command", 10, 10, 2);

tft.drawString("Baud Rate: 115200", 10, 30, 2);

}

void loop() {

// Check if a Serial command is available

if (Serial.available()) {

String command = Serial.readStringUntil('\n');

command.trim();

if (command.equals(START_COMMAND)) {

// Execute the draw function only after the command is received

drawImageFromSerial();

tft.drawString("Draw Complete!", 10, 50, 2);

} else {

Serial.printf("Received unknown command: %s\n", command.c_str());

}

}

}

## 4. Final Python Script (image_sender.py)

Save this script on your laptop in the same directory as your myimage.raw file.

Python

import serial

import time

import os

# --- Configuration ---

COM_PORT = 'COM17'

BAUD_RATE = 115200 # Must match the Arduino sketch

FILE_PATH = "myimage.raw"

START_COMMAND = "START_IMAGE_TRANSFER\n" # Command followed by newline

def send_image_serial():

if not os.path.exists(FILE_PATH):

print(f"Error: File not found at {FILE_PATH}. Make sure it is in the same directory as this script.")

return

try:

# Read the raw binary file content

with open(FILE_PATH, 'rb') as f:

raw_data = f.read()

expected_size = 320 * 170 * 2

if len(raw_data) != expected_size:

print(f"Error: File size mismatch. Expected {expected_size} bytes, got {len(raw_data)} bytes.")

return

print(f"Connecting to {COM_PORT} at {BAUD_RATE}...")

# Initialize serial connection

ser = serial.Serial(COM_PORT, BAUD_RATE)

ser.write_timeout = 1

time.sleep(2)

# STEP 1: Send the start command

print(f"Sending start command: {START_COMMAND.strip()}")

ser.write(START_COMMAND.encode())

time.sleep(0.5)

# STEP 2: Send raw image data

print(f"Sending {len(raw_data)} bytes of image data over Serial...")

bytes_sent = ser.write(raw_data)

ser.close()

if bytes_sent == expected_size:

print("--- SUCCESS ---")

print("Image transmission successful.")

else:

print(f"--- FAILURE --- (Only sent {bytes_sent} of {expected_size})")

except serial.SerialException as e:

print("--- SERIAL CONNECTION ERROR ---")

print(f"Could not open or write to port {COM_PORT}. Error: {e}")

print("Make sure the Serial Monitor in the Arduino IDE is closed.")

except Exception as e:

print(f"An unexpected error occurred: {e}")

if __name__ == "__main__":

send_image_serial()

## 5. Execution Steps

- Upload the ESP32 Sketch using the Arduino IDE.

- Close the Arduino Serial Monitor. (This is mandatory, as Python cannot access COM17 if the IDE has it open).

- Run the Python script in your PowerShell or Command Prompt:

Bash

python image_sender.py

- The script will send the command and then the image data, causing your image to display on the LCD.